CPE301 – SPRING 2019

# MIDTERM II

Student Name: Prachi Patel

Student #: 5002380222

Student Email: patelp3@unlv.nevada.edu

Primary Github address: https://github.com/prachi173

Directory: https://github.com/prachi173/da_sp18

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/Midterm, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmega328p XPlained Mini

APDS 9960

ESP-01

- DEVELOPED MODIFIED CODE OF TASK

#ifndef F_CPU

#define F_CPU 16000000UL

#endif

#define BAUD 9600

#define MYUBRR F_CPU/16/BAUD-1

#include <avr/io.h>

#include <avr/interrupt.h>

#include <util/delay.h>

#include <math.h>

#include <stdlib.h>

#include <stdio.h>

// include i2c, uart and APDS9960 library files

// APDS9960 library file was edited to add the following

//#define APDS_WRITE			(0x39 << 1) | 0

//#define APDS_READ			(0x39 << 1) | 1

#include "i2c_master.h"

#include "i2c_master.c"

#include "uart.h"

#include "uart.c"

#include "APDS9960_def.h"

void color_rgb();

void APDS9960_init();

uint8_t baud_rate = MYUBRR;

uint8_t RL, RH;

uint8_t BL, BH;

uint8_t GL, GH;

uint8_t config;

char outs[255];

int main(void){

char buffer[20], float_[10];

uint8_t red = 0;

uint8_t green = 0;

uint8_t blue = 0;

init_uart(9600); //initialize UART

_delay_ms(500);

USART_SendString("We are live!"); //message to confirm uart is working

_delay_ms(5000);

i2c_init(); //initialize i2c

APDS9960_init(); //initialize sensor

unsigned char AT_CHECK[] = "AT\r\n"; //AT command working check

unsigned char CWMODE_SET[] = "AT+CWMODE=1\r\n"; //Mode set to 1

unsigned char CWJAP_LOGIN[] = "AT+CWJAP=\"mywifi\",\"mypass\"\r\n"; //login personal wifi

unsigned char CIPMUX_SET[] = "AT+CIPMUX=0\r\n"; // CIPMUX set to 0

unsigned char CIPSTART_SEND[] = "AT+CIPSTART=\"TCP\",\"api.thingspeak.com\",80\r\n"; //connect to thingspeak

unsigned char CIPSEND_FINISH[] = "AT+CIPSEND=100\r\n"; //end communication

//send strings to print and delays added to establish proper communication

_delay_ms(3000);

USART_SendString(AT_CHECK);

_delay_ms(10000);

USART_SendString(CWMODE_SET);

_delay_ms(10000);

USART_SendString(CWJAP_LOGIN);

_delay_ms(20000);

while(1){

//send strings to print and delays added to establish proper communication

USART_SendString(CIPMUX_SET);

_delay_ms(100000);

USART_SendString(CIPSTART_SEND);

_delay_ms(10000);

USART_SendString(CIPSEND_FINISH);

_delay_ms(10000);

color_rgb(&red, &blue, &green);

//print to update field1 with red, field2 with green and field3 with blue

snprintf(outs,sizeof(outs),"GET https://api.thingspeak.com/update?api_key=7JVWL1NQ7916EIRM&field1=0%05u&field2=%05u&field3=%05u\r\n", red,green,blue);

USART_SendString(outs);

_delay_ms(10000);

}

return 0;

}

void color_rgb(uint8_t *red, uint8_t *blue, uint8_t *green){

//read colors

i2c_readReg(APDS_WRITE, APDS9960_RDATAL, &RL, 1);

i2c_readReg(APDS_WRITE, APDS9960_RDATAH, &RH, 1);

i2c_readReg(APDS_WRITE, APDS9960_GDATAL, &GL, 1);

i2c_readReg(APDS_WRITE, APDS9960_GDATAH, &GH, 1);

i2c_readReg(APDS_WRITE, APDS9960_BDATAL, &BL, 1);

i2c_readReg(APDS_WRITE, APDS9960_BDATAH, &BH, 1);

*red = RH << 8 | RL;

*blue = BH << 8 | BL;

*green = GH << 8 | GL;

}

void APDS9960_init(){

//reading and writing colors to register

i2c_readReg(APDS_WRITE, APDS9960_ID, &config,1);

if(config != APDS9960_ID_1)

while(1)

{

config = 1 << 1 | 1 << 0 | 1 << 3 | 1 << 4;

}

i2c_writeReg(APDS_WRITE, APDS9960_ENABLE, &config, 1);

config = DEFAULT_ATIME;

i2c_writeReg(APDS_WRITE, APDS9960_ATIME, &config, 1);

config = DEFAULT_WTIME;

i2c_writeReg(APDS_WRITE, APDS9960_WTIME, &config, 1);

config = DEFAULT_PROX_PPULSE;

i2c_writeReg(APDS_WRITE, APDS9960_PPULSE, &config, 1);

config = DEFAULT_POFFSET_UR;

i2c_writeReg(APDS_WRITE, APDS9960_POFFSET_UR, &config, 1);

config = DEFAULT_POFFSET_DL;

i2c_writeReg(APDS_WRITE, APDS9960_POFFSET_DL, &config, 1);

config = DEFAULT_CONFIG1;

i2c_writeReg(APDS_WRITE, APDS9960_CONFIG1, &config, 1);

config = DEFAULT_PERS;

i2c_writeReg(APDS_WRITE, APDS9960_PERS, &config, 1);

config = DEFAULT_CONFIG2;

i2c_writeReg(APDS_WRITE, APDS9960_CONFIG2, &config, 1);

config = DEFAULT_CONFIG3;

i2c_writeReg(APDS_WRITE, APDS9960_CONFIG3, &config, 1);

}

void init_uart(uint16_t baudrate){

//initialize uart

uint16_t UBRR_val = (F_CPU/16)/(baudrate-1);

UBRR0H = UBRR_val >> 8;

UBRR0L = UBRR_val;

UCSR0B |= (1<<TXEN0) | (1<<RXEN0);

UCSR0C |= (1<<USBS0) | (3<<UCSZ00);

}

void uart_outchar(unsigned char c){

//output uart data

while(!(UCSR0A & (1<<UDRE0))); // wait until sending is possible

UDR0 = c; // output character saved in c

}

void uart_outstring(char *s){

while(*s){

uart_outchar(*s);

s++;

}

}

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

For all 10 feeds

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://youtu.be/HJeCSGr0u58

- GITHUB LINK OF THIS DA

https://github.com/prachi173/da_sp18/tree/master/Midterms/MIDTERM2

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

PRACHI PATEL